Directions for Operating the Magnavox VHS to DVD Duplicator

These directions are made to provide a step by step procedure for recording VHS tapes to DVDs. They are set under sequence headers, if your current situation does not conform to the header do not follow any directions following it.

Required Materials

- VHS(s) to be recorded

- DVD-R

  - Not DVD-RW or any other type, except the one specified

Duplicating VHS content to a DVD

- Put a blank DVD-R into the DVD slot

- Wait for it to load

- Put the VHS you wish you duplicate into the VHS slot

- Advance the VHS to a few seconds before the point you wish to start duplicating and press pause

- Press the ‘dubbing’ button on the remote control

  - VHS will play at normal play speed

- When you reach the point you wish to stop recording hit stop

  - Also if there is a section of tape without data greater than 3 minutes it will automatically stop

- Please look at the following situations and follow their direction based on whichever criterion fits your current situation

  - Disk ran out of space and automatically stopped duplicating VHS content

    - Magnavox player will automatically finalize disk, wait until a DVD disc menu appears to remove disk

  - Reached end of desired content before disk was full

    - If you wish to add additional data to this disk please proceed directly to ‘Adding additional VHS video to a DVD’ header and follow the directions below it

    - If you are satisfied with the amount of information you have now placed on the disk please proceed to ‘Finalizing disk for play on other DVD players’ header and follow the directions below

Adding additional VHS video to a DVD

- When the disc menu appears navigate down to the box beside ‘Empty Title’ using up/down arrows on either side of the ‘ok’ button the remote control, use ‘ok’ button to select

  - The current selection will be highlighted in yellow

  - You can also press the ‘return’ button if you press a button you didn’t mean to, this will effectively undo your last button press of ‘ok’

- Selecting the box next to ‘Empty Title’ will make another set of decisions appear [play, edit, overwrite]

- Select ‘overwrite’

- Please refer back to ‘Duplicating VHS content to a DVD’ header, step 3 as all following decisions will be the same.

Finalizing disk for play on other DVD players

Finalizing a DVD is the process of locking the DVD so that it can be played on other DVD players. Until the DVD is finalized it cannot be read by any other DVD player and has the potential to be overwritten.

- When the disk menu appears press the ‘stop’ button

- Then press the ‘setup’ button

  - This will cause a menu to appear whose last entry will be ‘disk edit’

- Navigate down to ‘disk edit’ using up/down arrows and select it using the ‘ok’ button

  - Remember if you accidently make a selection you did not mean to do, simply press the ‘return’ button to undo your last command

- Selecting disk edit will cause yet another menu to appear

  - The last entry of this is ‘Finalize Disk’

- Select ‘Finalize Disk’

  - It will ask you to confirm this decision, remember once you finalize a disk the data is locked and can no longer be edited

- Using up/down arrows press ‘yes’

  - This will finalize the disk

- Once disk menu reappears the disk is finished and can be read by any DVD player